Assignment 02_Task 01

Task 1:

Experimental Setup:

Used Third.cc as reference and modified following lines to complete task1.

Line 30: added this code. This code just pulls the position information from the mobility model and unconditionally logs the x and y position of the node

void

CourseChange (std::string context, Ptr<const MobilityModel> model)

{

Vector position = model->GetPosition ();

NS_LOG_UNCOND (context <<

" x = " << position.x << ", y = " << position.y);

}

Line 42: changed the number of nodes from 3 to 6

uint32_t nWifi = 6;

Line 44: added useRts declaration

bool useRts = true;

Line 70: added the code to force the nodes to utilize the RTS/CTS handshake procedure

if (useRts)

{

Config::SetDefault ("ns3::WifiRemoteStationManager::RtsCtsThreshold",

StringValue ("0"));

}

Line 76: Added code for wifi nodes creation.

NodeContainer wifiNodes;

wifiNodes.Create(nWifi);

Line 81: Setting default parameters in IEEE 802.11G standard

WifiHelper wifi;

wifi.SetStandard (WIFI_STANDARD_80211g);

Line 92: changed the station mode (sta) to Ad-hoc mode

mac.SetType ("ns3::AdhocWifiMac",

"Ssid", SsidValue (ssid));

Line 115: changed the dimensions of lower-left corner and upper-right corner from 50 to 100m  as mentioned

mobility.SetMobilityModel ("ns3::RandomWalk2dMobilityModel",

"Bounds", RectangleValue (Rectangle (-100, 100, -100, 100)));

Line 128: Assigned the Ip address as 192.168.1.0

address.SetBase ("192.168.1.0", "255.255.255.0");

Line 132-135: changed the UDP Echo Server port to port 20 and added server node information.

UdpEchoServerHelper echoServer (20);

ApplicationContainer serverApps = echoServer.Install(wifiNodes.Get(0));

serverApps.Start(Seconds(1.0));

serverApps.Stop(Seconds(10.0));

Line 138-145: Added client 1 code at node 5

UdpEchoClientHelper echoClient(wifiInterfaces.GetAddress(0), 20);

echoClient.SetAttribute("MaxPackets", UintegerValue(2));

echoClient.SetAttribute("Interval", TimeValue(Seconds(2.0)));

echoClient.SetAttribute("PacketSize", UintegerValue(1024));

ApplicationContainer clientApps = echoClient.Install(wifiNodes.Get(5));

clientApps.Start(Seconds(2.0));

clientApps.Stop(Seconds(10.0));

Line 148-155: Added client 2 code  at node 4

UdpEchoClientHelper echoClient2(wifiInterfaces.GetAddress(0), 20);

echoClient2.SetAttribute("MaxPackets", UintegerValue(2));

echoClient2.SetAttribute("Interval", TimeValue(Seconds(1.0)));

echoClient2.SetAttribute("PacketSize", UintegerValue(1024));

ApplicationContainer clientApps2 = echoClient2.Install(wifiNodes.Get(4));

clientApps2.Start(Seconds(3.0));

clientApps2.Stop(Seconds(10.0));

Line 159-164: Added code for packet tracing at Node 2.

phy.SetPcapDataLinkType (WifiPhyHelper::DLT_IEEE802_11_RADIO);

phy.EnablePcap (useRts ? "rtscts-AT21" : "nortscts-AT21", staDevices.Get (2), true);

std::ostringstream oss;

oss <<"/NodeList/" << wifiNodes.Get (2)->GetId ()

<<"/$ns3::MobilityModel/CourseChange";

Config::Connect (oss.str (), MakeCallback (&CourseChange));

- Are all the frames acknowledged? Explain why?

No, All the frames are not acknowledged as the frames transmitted to a broadcast destination have a duration of 0 seconds. These frames are not acknowledged by the receivers as they are not part of t exchange.

2.  Are there any collisions in the network? Explain why. How have you reached this conclusion?

As per the screenshot that is attached below, there is a collision between Frames 11 and 12 in the network. Both of these frames wait for a seemingly arbitrary length of time before being sent again allows us to detect a collision.

3. How can you force the nodes to utilize the RTS/CTS handshake procedure seen in class? What is the reasoning behind this procedure?

We can use the following sample code to implement RTS/CTS in the ns-3 program:

Config::SetDefault ("ns3::WifiRemoteStationManager::RtsCtsThreshold", StringValue ("0"));

We can force the nodes to utilize this handshake procedure by setting the RTS/CTS packet size threshold to zero.

In wireless networks, when two frames sent from different nodes A and C to B, they may collide at B , but A and C will be unaware of collision due to hidden terminal problem. Hence, RTS/CTS handshake procedure is used to avoid the collision instead of detecting it.

4. Force the utilization of RTS/CTS in the network:

a. Are there any collisions now?

After implementation of RTS/CTS methodology, no collisions were observed                                   b. Which is the benefit of RTS/CTS?

RTS/CTS helps to avoid collision that occurs due to Hidden Terminal (nodes).

c. Where can you find the Network Allocation Vector information?

Duration field contains the Network Allocation Vector information. Access to the medium is restricted for the time specified by the NAV.

Output of task1 without RTS/CTS

Output of task1 with forcing the utilization RTS/CTS